# Embedded Systems Design Report

Menu Driven RGB LED Controller using TM4C123

Uma Bharathi D (SR: 25964)

Branch: DESE

## 1. Objective

To design and implement a menu-driven RGB LED controller using the TM4C123GH6PM microcontroller. The system uses push buttons for menu navigation, SysTick timer for time control and display multiplexing, and a 4-digit 7-segment display for user interface.

## 2. Design Overview

The system consists of three major modules: 1) Switch and RGB LED interface using Port F, 2) 7-segment display multiplexing using Ports A and B, 3) SysTick timer interrupt for timing control and display refresh.

## 3. Functional Description

• Two switches (SW1 and SW2) are used for menu navigation.
• Menu states: SC (Select Color), SP (Select Speed), EN (Enable), ST (Stop), RS (Reset).
• RGB LED color is selected using a 3-bit value mapped to PF1–PF3.
• Blinking speed is controlled using predefined timing values stored in blink_table[].
• SysTick interrupt runs at 1ms interval for multiplexing and blink timing.
• 4-digit 7-segment display shows current menu state and selected parameters.

## 4. Implementation Details

• PortF_Init(): Configures switches and RGB LED pins with interrupts.
• Seg7_Init(): Initializes segment lines on Port B and digit select lines on Port A.
• SysTick_Init(): Configured for 1ms interrupt at 16MHz clock.
• GPIOPortF_Handler(): Handles switch interrupts for state changes.
• SysTick_Handler(): Performs display multiplexing and blink timing.
• LED_Task(): Controls RGB LED output based on state and blink toggle.

## 5. Observations

• Menu navigation works reliably using external interrupts.
• 7-segment multiplexing is stable without flicker.
• Blink frequency changes correctly according to selected speed.
• Reset state restores default color and speed values.

## 6. Conclusion

A menu-driven RGB LED controller was successfully implemented using TM4C123. The system demonstrates the use of GPIO interrupts, SysTick timer, display multiplexing, and structured state-machine based embedded design. The design is modular and can be extended for additional embedded applications.