# Lab 06 – Interrupt Driven Control System

Course: Embedded Control Systems Design
Student: Uma Bharathi D
SR Number: 25964
Institute: DESE, Indian Institute of Science, Bangalore

# 1. Objective

The objective of this experiment is to design and implement an interrupt-driven embedded control system that synchronizes both physical hardware inputs (switches) and serial software inputs (UART commands). The system must allow users to cycle through different operational modes and modify configuration parameters such as RGB LED color and blinking speed.

The system should also ensure proper interlocking mechanisms so that hardware switches and UART commands do not interfere with each other, thereby avoiding ambiguous configuration states.

# 2. System Description

The implemented system operates using five operational modes:

| Mode | Description |
| --- | --- |
| SC | Set Colour |
| SP | Set Speed |
| EN | Enable blinking |
| ST | Stop blinking |
| RS | Resume / Reset configuration |

Two onboard switches and UART commands are used to control the system:

- Switch 2 (PF0) – Cycles through the operational modes.

- Switch 1 (PF4) – Increments configuration indices such as color or speed.

Similarly, UART commands allow remote control of the same functionality.

# 3. Operational Logic

The RGB LED connected to Port F pins (PF1–PF3) supports 8 color combinations using the RGB bit combinations. These colors are stored in a lookup table.

Similarly, the blinking speed is defined using 8 delay values ranging from:

- 2000 ms (2 seconds)

- 100 ms

A SysTick timer interrupt running every 1 ms is used to control the blinking timing. The timer increments a counter which toggles the LED once the desired delay threshold is reached.

# 4. Interrupt Driven Architecture

The system uses three interrupt sources:

### 1. GPIO Interrupt (Switch Input)

GPIO interrupts detect falling-edge triggers when switches are pressed.
The interrupt handler performs:

- Mode cycling (SW2)

- Configuration index increment (SW1)

- Reset or enable operations depending on current mode

Debounce logic is implemented using a software counter updated inside SysTick.

### 2. UART Interrupt

UART receives commands from a serial terminal.

Commands supported:

| Command | Function |
| --- | --- |
| c | Enter UART configuration mode |
| x | Exit UART configuration mode |
| m | Cycle operational mode |
| i | Increment configuration index |

Incoming characters are echoed back to the terminal.

3. SysTick Interrupt

The SysTick timer generates 1 ms periodic interrupts and performs:

- Switch debounce handling

- LED blinking control

- Timing synchronization

### 7. Conclusion

This experiment demonstrates the effectiveness of interrupt-driven embedded system design. By combining GPIO interrupts, UART communication, and SysTick timers, a responsive and robust control system was implemented.

The design also illustrates how multiple input sources can be synchronized using proper interlocking mechanisms, which is an important concept in embedded system reliability.