# UART Based LCD & LED Control System using State Machine

Name: Uma Bharathi D

SR Number: 25964

Programme: M.Tech ESE

Course: ESD-I

## 1. Objective

To design and implement a UART-based command parser that controls a 16x2 LCD and the onboard LED using a State Machine architecture without using interrupts or blocking delays.

## 2. Hardware Configuration

• Microcontroller: TM4C123GH6PM (Tiva C)

• LCD 1602A (8-bit mode)

• UART0 (9600 baud serial communication)

• Onboard RED LED (PF1)

LCD Connections:

• Data Bus (D0–D7) → PB0–PB7

• RS → PA6

• E → PA7

• R/W → GND (Write mode only)

## 3. System Architecture

The system is implemented using a cooperative State Machine approach. The main loop continuously executes independent tasks without blocking execution.

Main Loop Tasks:

• UART_Task() – Receives characters and builds command string.

• LED_Task() – Handles ON/OFF/BLINK states non-blocking.

• LCD_Task() – Handles static display and scrolling states.

• tick counter – Used for time-based operations.

## 4. Supported Commands

ON      → Turn LED ON

OFF     → Turn LED OFF

BLINK   → Non-blocking LED toggle

PRINT <msg> → Display message on LCD (stops scrolling)

FWD     → Scroll LCD forward

BWD     → Scroll LCD backward

CLEAR   → Clear LCD and stop motion

## 5. State Machine Design

LED States: LED_OFF, LED_ON, LED_BLINK

LCD States: LCD_IDLE, LCD_STATIC, LCD_SCROLL_FWD, LCD_SCROLL_BWD

State transitions occur when valid UART commands are received. Each state is evaluated inside its respective task function.

## 6. Key Embedded Concepts Used

• Non-blocking timing using tick counters

• Finite State Machine implementation

• UART command parsing

• Modular firmware design

• Hardware abstraction and separation of concerns

## 7. Conclusion

The project demonstrates structured embedded firmware design using state machines. It handles simultaneous LED blinking and LCD scrolling without interrupts or blocking delays, ensuring responsive UART command control.